SIMATS ENGINEERING

ASSESSMENT TOOL 1

# COURSE NAME: 	CSA07 COURSE CODE: Computer Networks

# COURSE OUTCOME COVERED

CO1: Apply the functions of OSI layer in enabling reliable data transmission across networks. (BL2, BL3)

Assessment Tool Weightage: 40%

Annexure A

SHORT ANSWER TEST

Code of Conduct:

I SANDHIYA S (192521224) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

SANDHIYA S

Annexure A

# Short Answer Test

- A company needs to transfer a 150 MB design file from a client computer to a remote server over a network with a bandwidth of 50 Mbps. The Transport Layer divides the file into 1500-byte segments, while the Data Link Layer encapsulates each segment into frames by adding 40 bytes of header and trailer. Calculate the total number of segments and frames transmitted, the total transmission overhead introduced by the Data Link Layer, and estimate the total transmission time (ignoring propagation delay). Explain how the Transport and Data Link layers ensure reliable communication.

- A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions.

Explain how flow control improves communication efficiency.

- A 12,000-byte IP packet is transmitted through a router where the Maximum Transmission Unit (MTU) is 1500 bytes. Assuming each fragment requires a 20byte IP header, calculate the total number of fragments created and the total header overhead introduced. Explain how the Network Layer ensures successful delivery of fragmented packets to the destination.

- A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions.

Explain how flow control improves communication efficiency.

- A multimedia application transfers a 600 MB video file. The Session Layer inserts a synchronization checkpoint after every 60 MB of transmitted data. During transmission, a failure occurs after 390 MB has been transferred. Determine the number of checkpoints created before failure and calculate the amount of data that must be retransmitted after recovery. Explain the importance of synchronization in the Session Layer.

- Before transmission, the Presentation Layer compresses a 900 MB file by 40%. The compressed data is encrypted, adding 5% overhead to the compressed size. Calculate the compressed file size, the encrypted file size, and the total percentage reduction compared to the original file. Explain how the Presentation Layer improves transmission efficiency and security.

7.An FTP application transfers 3 GB of data over a network with an effective throughput of 120 Mbps. The Application Layer divides the data into requests of 3 MB each. Calculate the total number of requests generated and estimate the total transmission time in seconds. Discuss how the Application Layer provides reliable services to users.

8.A packet experiences the following processing delays at different OSI layers:

Application Layer = 4 ms, Transport Layer = 3 ms, Network Layer = 5 ms, Data Link Layer = 2 ms, and Physical Layer = 1 ms. The propagation delay is 18 ms, and the transmission delay is 12 ms. Calculate the total end-to-end delay experienced by the packet and explain how each layer contributes to the communication process.

9.A network transfers 80 MB of useful data using frames of 1600 bytes, where each frame contains 80 bytes of protocol overhead added by different OSI layers.

Calculate the total number of frames required, the total overhead transmitted, and the transmission efficiency as a percentage. Explain how protocol overhaffects network performance.

10.A hospital transfers a patient's medical record of 240 MB through an IoTenabled healthcare network. The Presentation Layer compresses the data by 30%, the Transport Layer divides the compressed data into 1400-byte segments, and the Data Link Layer adds 60 bytes of overhead to each frame. Calculate the compressed file size, the total number of segments transmitted, the total protocol overhead introduced, and the final amount of data transmitted across the Physical Layer. Explain how multiple OSI layers work together to ensure secure and

reliable communication.

ANSWERS

1. Given

- File size: 150 MB = 150,000,000 bytes

- Transport layer segment size: 1500 bytes

- Data Link layer overhead per frame: 40 bytes

- Bandwidth: 50 Mbps = 50,000,000 bits/s

Number of Transport Layer Segments

Number of segments=150,000,0001500=100,000

Segments = 100,000

Number of Data Link Layer Frames

Each transport segment is encapsulated into one frame.

Frames transmitted = 100,000

Total Data Link Layer Overhead

Each frame adds 40 bytes of header and trailer.

100,000×40=4,000,000 bytes

Total overhead = 4,000,000 bytes (4 MB)

Total Data Transmitted

150,000,000+4,000,000=154,000,000 bytes

Convert to bits:

154,000,000×8=1,232,000,000 bits

Transmission Time

Time=1,232,000,00050,000,000=24.64 seconds

Estimated transmission time = 24.64 seconds (ignoring propagation delay)

| Parameter | Value |
| --- | --- |
| File size | 150 MB |
| Segment size | 1500 bytes |
| Number of segments | 100,000 |
| Number of frames | 100,000 |
| Header/trailer per frame | 40 bytes |
| Total overhead | 4,000,000 bytes (4 MB) |
| Total transmitted data | 154,000,000 bytes |
| Transmission time | 24.64 seconds |

How the Transport and Data Link Layers Ensure Reliable Communication

Transport Layer (e.g., TCP):

- Divides the file into manageable segments.

- Assigns sequence numbers so data can be reassembled correctly.

- Uses acknowledgments (ACKs) to confirm successful delivery.

- Retransmits lost or corrupted segments.

- Performs flow control to prevent overwhelming the receiver.

- Uses error checking (checksums) to detect data corruption.

Data Link Layer:

- Encapsulates each segment into a frame by adding headers and trailers.

- Uses Frame Check Sequence (FCS)/CRC to detect transmission errors.

- Ensures reliable delivery over a single network link.

- May retransmit corrupted frames (depending on the protocol).

- Controls access to the transmission medium using MAC addressing and media access mechanisms.

Together, the Transport Layer provides end-to-end reliability across the network, while the Data Link Layer ensures reliable communication over each individual network link by detecting and handling frame errors.

# 2.  Given

# Sliding window size = 6 frames

# Data per frame = 1024 bytes

# Total frames to transmit = 48 frames

# One frame is lost in every window and must be retransmitted

# Number of Transmission Windows Required

# The number of windows is:

# Number of windows=Total framesWindow size
=486=8

# Number of transmission windows = 8

# Total Number of Retransmissions

# Since one frame is lost in every window, and there are 8 windows:

# Retransmissions=8×1=8

# Total retransmissions = 8 frames

# Total Frames Sent Including Retransmissions

# 48+8=56 frames

# Total frames transmitted = 56 frames

| # Parameter | # Value |
| --- | --- |
| # Total frames | # 48 |
| # Window size | # 6 frames |
| # Number of windows | # 8 |
| # Lost frames per window | # 1 |
| # Total retransmissions | # 8 frames |
| # Total frames sent (including retransmissions) | # 56 frames |
| # Data per frame | # 1024 bytes |

# How Flow Control Improves Communication Efficiency

# Flow control is a mechanism used by the Transport Layer to regulate the rate of data transmission between the sender and receiver.

# Benefits of Flow Control

# Prevents Receiver Overload

# Ensures the sender does not send data faster than the receiver can process or store it.

# Reduces Packet Loss

# Prevents buffer overflow at the receiver, reducing dropped frames and retransmissions.

# Improves Network Utilization

# Maintains a steady flow of data instead of frequent pauses and retransmissions.

# Enhances Throughput

# Allows multiple frames to be sent before waiting for acknowledgments, increasing efficiency.

# Supports Reliable Communication

# Works with acknowledgments and window management to ensure data is delivered correctly and in order.

# Sliding Window Flow Control

# With a window size of 6, the sender can transmit up to 6 frames before needing acknowledgments. As ACKs arrive, the window "slides" forward, allowing new frames to be sent. This keeps the communication channel busy and improves overall transmission efficiency compared to sending one frame at a time.

# Conclusion: Flow control balances the transmission rate between sender and receiver, minimizes congestion and retransmissions, and increases the overall efficiency and reliability of data communication.

3. Given

- Original IP packet size = 12,000 bytes

- MTU = 1,500 bytes

- IP header per fragment = 20 bytes

Maximum Data per Fragment

Each fragment must fit within the MTU, including its IP header.

Data per fragment=1500-20=1480 bytes

Number of Fragments

Assuming the 12,000-byte packet includes the original header, the payload is:

12000-20=11980 bytes

Number of fragments:

119801480=8.0946=9

Total fragments created = 9

Total Header Overhead

Each fragment requires its own 20-byte IP header.

9×20=180 bytes

Total header overhead after fragmentation = 180 bytes

If we consider that the original packet already contained one 20-byte header, the additional overhead introduced by fragmentation is:

(9-1)×20=160 bytes

| Parameter | Value |
| --- | --- |
| Original packet size | 12,000 bytes |
| MTU | 1,500 bytes |
| IP header per fragment | 20 bytes |
| Data carried per fragment | 1,480 bytes |
| Number of fragments | 9 |
| Total fragment headers | 180 bytes |
| Additional overhead due to fragmentation | 160 bytes |

How the Network Layer Ensures Successful Delivery of Fragmented Packets

The Network Layer (IP) handles fragmentation and reassembly so that packets can traverse networks with different MTU sizes.

1. Fragmentation

- When a packet is larger than the outgoing link's MTU, the router divides it into smaller fragments.

- Each fragment receives its own IP header.

2. Identification Field

- All fragments of the same original packet carry the same Identification value.

- This allows the destination to recognize which fragments belong together.

3. Fragment Offset

- Each fragment contains a Fragment Offset indicating its position within the original packet.

- The destination uses these offsets to place fragments in the correct order.

4. More Fragments (MF) Flag

- The MF flag indicates whether more fragments follow.

- The last fragment has MF = 0, signaling the end of the packet.

5. Routing

- Fragments are routed independently through the network.

- They may take different paths and arrive out of order.

6. Reassembly

- Only the destination host reassembles the fragments.

- Using the Identification field, Fragment Offset, and MF flag, it reconstructs the original 12,000-byte packet.

7. Error Handling

- If one or more fragments are lost, the complete packet cannot be reassembled.

- Higher-layer protocols such as TCP detect the loss and trigger retransmission.

Conclusion: The Network Layer enables large packets to traverse networks with smaller MTUs by fragmenting them into 9 fragments, adding 180 bytes of fragment headers (160 bytes of extra overhead due to fragmentation), and using identification, offsets, and flags to ensure correct reassembly at the destination.

4. Given Data

- Sliding window size = 6 frames

- Data carried by each frame = 1024 bytes

- Total frames transmitted = 48 frames

- One frame is lost in every window and must be retransmitted

Number of Transmission Windows Required

The number of windows required is:

Number of windows=Total framesWindow size
=486
=8

Number of transmission windows = 8

Total Number of Retransmissions

Since one frame is lost in every window, and there are 8 windows:

Total retransmissions=8×1
=8

Total retransmissions = 8 frames

Total Data Originally Sent

48×1024=49,152 bytes

Original user data transmitted = 49,152 bytes

Total Data Retransmitted

8×1024=8,192 bytes

Retransmitted data = 8,192 bytes

| Parameter | Value |
| --- | --- |
| Total frames | 48 |
| Window size | 6 frames |
| Number of windows | 8 |
| Lost frames per window | 1 |
| Total retransmissions | 8 frames |
| Data per frame | 1024 bytes |
| Original data transmitted | 49,152 bytes |
| Retransmitted data | 8,192 bytes |

How Flow Control Improves Communication Efficiency

Flow control is a Transport Layer mechanism that regulates the amount of data a sender can transmit before receiving acknowledgments from the receiver.

Benefits of Flow Control

- Prevents Receiver Buffer Overflow

  - Ensures the sender does not send data faster than the receiver can process it.

- Reduces Data Loss

  - By avoiding congestion at the receiver, fewer frames are dropped and retransmitted.

- Improves Throughput

  - Multiple frames can be sent in one window before waiting for acknowledgments, making better use of available bandwidth.

- Efficient Use of Network Resources

  - Keeps the communication channel busy while minimizing idle time.

- Supports Reliable Transmission

  - Lost or corrupted frames can be detected and retransmitted without affecting the entire data transfer.

Sliding Window Example

With a window size of 6, the sender can transmit up to 6 frames before requiring acknowledgments. As ACKs arrive, the window slides forward, allowing continuous transmission. This is much more efficient than a stop-and-wait approach, where only one frame is sent at a time.

Conclusion

- Transmission windows required: 8

- Total retransmissions: 8 frames

- Flow control improves communication efficiency by matching the sender's transmission rate to the receiver's capacity, reducing congestion, minimizing retransmissions, and increasing overall throughput.

5. Given

- Video file size = 600 MB

- Checkpoint interval = 60 MB

- Failure occurs after = 390 MB transferred

Number of Checkpoints Created Before Failure

The Session Layer inserts a checkpoint every 60 MB:

| Data Transferred | Checkpoint |
| --- | --- |
| 60 MB | 1 |
| 120 MB | 2 |
| 180 MB | 3 |
| 240 MB | 4 |
| 300 MB | 5 |
| 360 MB | 6 |

The next checkpoint would be at 420 MB, but the failure occurs at 390 MB.

Therefore,

Number of checkpoints before failure=36060=6

Checkpoints created before failure = 6

Data to be Retransmitted After Recovery

After a failure, transmission resumes from the last successful checkpoint.

- Last checkpoint = 360 MB

- Failure occurred at = 390 MB

Data transmitted after the last checkpoint:

390-360=30 MB

This portion must be retransmitted.

Data to be retransmitted = 30 MB

| Parameter | Value |
| --- | --- |
| Total file size | 600 MB |
| Checkpoint interval | 60 MB |
| Failure point | 390 MB |
| Last completed checkpoint | 360 MB |
| Number of checkpoints created | 6 |
| Data to be retransmitted | 30 MB |

Importance of Synchronization in the Session Layer

The Session Layer (Layer 5 of the OSI model) manages communication sessions between applications and provides synchronization points (checkpoints) during long data transfers.

Benefits of Synchronization

- Reduces Retransmission

  - After a failure, communication resumes from the last checkpoint rather than restarting from the beginning.

- Improves Reliability

  - Ensures long file transfers, multimedia streams, and database transactions can recover efficiently from interruptions.

- Saves Time and Bandwidth

  - Only the data sent after the most recent checkpoint must be retransmitted.

- Supports Error Recovery

  - Checkpoints act as recovery markers that help restore communication sessions.

- Enhances Performance

  - Particularly useful for large files and long-running sessions where restarting the entire transfer would be costly.

Conclusion

For the 600 MB video transfer:

- 6 synchronization checkpoints are created before the failure.

- Only 30 MB of data must be retransmitted after recovery.

- Session Layer synchronization improves efficiency, reliability, and recovery by allowing communication to resume from the latest checkpoint instead of retransmitting the entire file.

6. Given

- Original file size = 900 MB

- Compression = 40%

- Encryption overhead = 5% of the compressed size

1. Compressed File Size

A 40% compression means the file size is reduced by 40%.

Compressed size=900×(1-0.40)
=900×0.60
=540 MB

Compressed file size = 540 MB

Encrypted File Size

Encryption adds 5% overhead to the compressed size.

Encryption overhead=540×0.05
=27 MB
Encrypted file size=540+27
=567 MB

Encrypted file size = 567 MB

Total Percentage Reduction Compared to the Original File

Reduction in size:

900-567=333 MB

Percentage reduction:

333900×100
=37%

Total percentage reduction = 37%

| Parameter | Value |
| --- | --- |
| Original file size | 900 MB |
| Compression reduction | 40% |
| Compressed file size | 540 MB |
| Encryption overhead | 27 MB |
| Encrypted file size | 567 MB |
| Net size reduction | 333 MB |
| Total percentage reduction | 37% |

How the Presentation Layer Improves Transmission Efficiency and Security

The Presentation Layer (Layer 6 of the OSI model) is responsible for data representation, compression, encryption, and translation between different data formats.

1. Improving Transmission Efficiency

- Compression reduces the amount of data that must be transmitted.

- Smaller files require:

  - Less bandwidth

  - Shorter transmission time

  - Lower storage requirements

- In this example, the file size is reduced from 900 MB to 540 MB before transmission.

2. Improving Security

- Encryption converts readable data (plaintext) into an unreadable form (ciphertext).

- Protects data from unauthorized access during transmission.

- Only authorized users with the correct decryption key can recover the original data.

3. Data Format Translation

- Converts data between different character encoding and data formats.

- Ensures compatibility between heterogeneous systems.

4. Data Integrity and Privacy

- Encryption helps maintain confidentiality.

- Prevents attackers from understanding intercepted data.

Conclusion

- Compressed file size: 540 MB

- Encrypted file size: 567 MB

- Overall reduction compared to the original file: 37%

The Presentation Layer enhances efficiency through compression and security through encryption, enabling faster and safer data transmission across networks.

7. Given

- Total data to transfer = 3 GB = 3000 MB (using decimal units)

- Request size = 3 MB

- Effective throughput = 120 Mbps

Total Number of Requests Generated

Number of requests=Total dataRequest size
=3000 MB3 MB
=1000

Total requests generated = 1000

Total Transmission Time

First, convert the total data size to bits:

3000 MB×8=24000 Mb

Using the throughput:

Transmission Time=Total Data (Mb)Throughput (Mbps)
=24000120
=200 seconds

Estimated transmission time = 200 seconds

(approximately 3 minutes 20 seconds)

| Parameter | Value |
| --- | --- |
| Total data size | 3 GB (3000 MB) |
| Request size | 3 MB |
| Number of requests | 1000 |
| Throughput | 120 Mbps |
| Total data in megabits | 24,000 Mb |
| Transmission time | 200 seconds (≈ 3 min 20 s) |

How the Application Layer Provides Reliable Services to Users

The Application Layer (Layer 7 of the OSI model) is the layer closest to the end user and provides network services directly to applications such as FTP, HTTP, SMTP, and DNS.

1. User-Friendly Network Access

- Provides interfaces that allow users and applications to access network resources without dealing with lower-layer details.

2. File Transfer Services

- Protocols such as FTP manage file uploads and downloads.

- Large files are divided into manageable requests or data blocks for efficient transfer.

3. Error Reporting and Recovery

- Detects transfer failures and informs the user.

- Supports restarting interrupted transfers and retransmitting missing data.

4. Authentication and Authorization

- Verifies user credentials before granting access to files and services.

- Protects network resources from unauthorized users.

5. Data Integrity

- Works with lower layers to ensure files arrive completely and correctly.

- FTP can verify file size and transfer status to confirm successful delivery.

6. Service Management

- Establishes, maintains, and terminates application sessions.

- Provides status information, acknowledgments, and progress monitoring.

Conclusion

For the FTP transfer:

- Number of requests generated: 1000

- Estimated transmission time: 200 seconds (3 minutes 20 seconds)

The Application Layer ensures reliable service by providing file transfer management, user authentication, error handling, session control, and an easy-to-use interface that allows users to transfer data efficiently and securely.

8.

Given

| Layer | Processing Delay |
| --- | --- |
| Application Layer | 4 ms |
| Transport Layer | 3 ms |
| Network Layer | 5 ms |
| Data Link Layer | 2 ms |
| Physical Layer | 1 ms |

Additional delays:

- Propagation delay = 18 ms

- Transmission delay = 12 ms

Total Processing Delay

Processing Delay=4+3+5+2+1
=15 ms

Total End-to-End Delay

Total Delay=Processing Delay+Transmission Delay+Propagation Delay
=15+12+18
=45 ms

Total End-to-End Delay = 45 ms

| Delay Type | Value |
| --- | --- |
| Application Layer delay | 4 ms |
| Transport Layer delay | 3 ms |
| Network Layer delay | 5 ms |
| Data Link Layer delay | 2 ms |
| Physical Layer delay | 1 ms |
| Total Processing Delay | 15 ms |
| Transmission Delay | 12 ms |
| Propagation Delay | 18 ms |
| Total End-to-End Delay | 45 ms |

Contribution of Each OSI Layer to Communication

1. Application Layer (4 ms)

- Provides network services directly to user applications.

- Supports protocols such as HTTP, FTP, SMTP, and DNS.

- Generates and receives user data.

2. Transport Layer (3 ms)

- Segments data into smaller units.

- Provides end-to-end communication.

- Handles flow control, error recovery, acknowledgments, and retransmissions (e.g., TCP).

3. Network Layer (5 ms)

- Determines the best route from source to destination.

- Performs logical addressing using IP addresses.

- Handles routing and packet forwarding.

4. Data Link Layer (2 ms)

- Encapsulates packets into frames.

- Uses MAC addresses for local delivery.

- Detects transmission errors using CRC/FCS.

5. Physical Layer (1 ms)

- Converts bits into electrical, optical, or radio signals.

- Handles the actual transmission medium and signal encoding.

Other Delays

Transmission Delay (12 ms)

- Time required to place all packet bits onto the communication link.

Transmission Delay=Packet SizeBandwidth

Propagation Delay (18 ms)

- Time taken for the signal to travel through the medium from sender to receiver.

Propagation Delay=DistancePropagation Speed

Conclusion

The packet experiences:

Total End-to-End Delay=45 ms

This delay consists of:

- 15 ms of processing across the OSI layers,

- 12 ms of transmission delay, and

- 18 ms of propagation delay.

Each OSI layer performs a specific function—ranging from application services to physical signal transmission—working together to ensure reliable and efficient communication.

9. Given

- Useful data = 80 MB = 80,000,000 bytes

- Frame size = 1600 bytes

- Protocol overhead per frame = 80 bytes

Therefore,

Useful data per frame=1600-80=1520 bytes

Total Number of Frames Required

Number of frames=80,000,0001520
=52631.58
=52,632

Total frames required = 52,632

Total Overhead Transmitted

Total overhead=52,632×80
=4,210,560 bytes

Total overhead transmitted = 4,210,560 bytes ≈ 4.21 MB

Transmission Efficiency

Transmission efficiency is the percentage of useful data in each frame.

Efficiency=Useful DataFrame Size×100
=15201600×100
=95%

Transmission efficiency = 95%

(Equivalently, overhead percentage = 801600×100=5%.)

| Parameter | Value |
| --- | --- |
| Useful data | 80 MB |
| Frame size | 1600 bytes |
| Overhead per frame | 80 bytes |
| Useful data per frame | 1520 bytes |
| Number of frames required | 52,632 |
| Total overhead transmitted | 4,210,560 bytes (≈ 4.21 MB) |
| Transmission efficiency | 95% |
| Overhead percentage | 5% |

How Protocol Overhead Affects Network Performance

Protocol overhead consists of headers, trailers, checksums, addresses, sequence numbers, and other control information added by various OSI layers.

Positive Effects

- Reliability: Enables error detection and recovery.

- Routing: Provides source and destination addressing.

- Flow Control: Prevents congestion and receiver overload.

- Security: Supports authentication and encryption.

- Session Management: Maintains communication state.

Negative Effects

- Consumes Bandwidth: Overhead occupies part of every frame, leaving less room for user data.

- Increases Transmission Time: More bits must be transmitted.

- Reduces Throughput: Effective data transfer rate becomes lower than the raw link speed.

- Adds Processing Load: Devices must generate, read, and process protocol headers.

Conclusion

For the transfer of 80 MB of useful data:

- Frames required: 52,632

- Total overhead transmitted: 4,210,560 bytes (≈ 4.21 MB)

- Transmission efficiency: 95%

Protocol overhead is essential for reliability, addressing, and control, but it also consumes bandwidth and slightly reduces overall network efficiency.

10. Given

- Original medical record size = 240 MB = 240,000,000 bytes

- Compression by Presentation Layer = 30%

- Transport Layer segment size = 1400 bytes

- Data Link Layer overhead per frame = 60 bytes

Compressed File Size

A 30% compression reduces the file size to 70% of the original.

Compressed Size=240,000,000×0.70
=168,000,000 bytes
=168 MB

Compressed file size = 168 MB

Total Number of Segments Transmitted

Number of Segments=168,000,0001400=120,000

Since it divides exactly:

Total segments transmitted = 120,000

Assuming one segment is encapsulated into one frame:

Total frames transmitted = 120,000

Total Protocol Overhead Introduced

Each frame adds 60 bytes of overhead.

Total Overhead=120,000×60
=7,200,000 bytes
=7.2 MB

Total protocol overhead = 7,200,000 bytes (7.2 MB)

Final Amount of Data Transmitted Across the Physical Layer

Final Data=Compressed Data+Overhead
=168,000,000+7,200,000
=175,200,000 bytes
=175.2 MB

Final data transmitted across the Physical Layer = 175.2 MB

| Parameter | Value |
| --- | --- |
| Original file size | 240 MB |
| Compression | 30% |
| Compressed file size | 168 MB |
| Segment size | 1400 bytes |
| Number of segments | 120,000 |
| Data Link overhead/frame | 60 bytes |
| Total overhead | 7,200,000 bytes (7.2 MB) |
| Final transmitted data | 175,200,000 bytes (175.2 MB) |

How Multiple OSI Layers Work Together for Secure and Reliable Communication

1. Presentation Layer (Layer 6)

- Compresses the medical record to reduce bandwidth usage and transmission time.

- May encrypt sensitive patient information to ensure confidentiality and privacy.

2. Transport Layer (Layer 4)

- Divides the compressed data into 1400-byte segments.

- Provides end-to-end reliability using sequence numbers, acknowledgments, error recovery, and flow control.

- Ensures all segments arrive correctly and in order.

3. Network Layer (Layer 3)

- Assigns logical addresses (IP addresses).

- Selects the best route through the healthcare network.

- Delivers packets from source to destination across multiple networks.

4. Data Link Layer (Layer 2)

- Encapsulates segments into frames.

- Adds 60-byte headers/trailers for addressing and error detection.

- Uses CRC/FCS to detect transmission errors on each link.

5. Physical Layer (Layer 1)

- Converts frames into electrical, optical, or wireless signals.

- Transmits the final 175.2 MB of data through the communication medium.

Conclusion

For the 240 MB medical record:

- Compressed file size: 168 MB

- Segments transmitted: 120,000

- Protocol overhead introduced: 7.2 MB

- Final data transmitted across the Physical Layer: 175.2 MB

The OSI layers collaborate to provide efficient, secure, and reliable communication, which is essential for transmitting critical healthcare information in IoT-enabled medical networks.

Top of Form

Bottom of Form

# RUBRICS FOR CALCULATION

| Criteria | Weightag e | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understandin g of Concepts | 25% | Demonstrates a thorough, accurate understandin g of concepts | Good understanding with minor gaps | Basic understanding with some misconception s | Poor understanding; major misconception s |
| Explanation | 25% | Detailed, wellexplained answers with relevant examples | Clear explanation with adequate details | Limited explanation; lacks depth | Poor or unclear explanation |
| Application | 20% | Effectively applies concepts with strong analytical ability | Applies concepts with minor errors | Limited application with weak analysis | No application or incorrect analysis |
| Organization | 15% | Wellstructured answers with logical flow | Organized with minor issues in flow | Basic structure, lacks clarity | Poor organization, incoherent answers |
|  |  | and coherence |  |  |  |
| Accuracy | 10% | Completely accurate and covers all required points | Mostly accurate with minor omissions | Partially correct with missing points | Incorrect answers with major gaps |
| Clarity | 5% | Clear, neat, professional presentation | Understandabl e with minor issues | Limited clarity, readability issues | Poorly written, difficult to understand |